PLANETARY DATA SYSTEM

USER'S GUIDE FOR THE PDS IMDISP PROGRAM

INTRODUCTION

This  document describes the IMDISP program, an  interactive

image  display utility for the IBM Personal Computer family  (PC,

XT  and AT) and compatibles. It has been developed for  use  with

the  Compact  Disk  - Read-Only Memory  (CDROM)  storage  systems

currently being evaluated by the Planetary Data System (PDS).  It

can  also  be  used to display images stored on  floppy  or  hard

disks.

This  User's Guide presents an overview of image  processing

and  CDROM fundamentals, a section on the installation of  IMDISP

and CDROM software and hardware, a description of IMDISP  program

operation,  a description of other utility programs which can  be

used in conjunction with IMDISP, and a command summary.

Appendices  are provided which describe the contents of  the

IMDISP distribution disk, the contents and format of the  Voyager

Uranus   and   Interactive  Data  Interchange  CDROM   disks,   a

description  of  the  data labeling  standards  and  image  file

formats  found on these CDROM disks, and references  for  further

information about image processing and CDROMs.

Other  support  software for using the PDS CDROM  disks  has

also  been developed. These packages include the  USGS  Planetary

Image Cartography System (PICS) for Microvax computers, a set  of

FORTRAN routines to access CDROM disks on VAX computers  (VMSCD),

and "C" language source code to process disks written in the High

Sierra  format. Contact Mike Martin (JPLPDS::MMARTIN) for  access

to these tools.

Chapter 2

BACKGROUND

Efforts  to utilize small computer systems for  display  and

analysis  of  scientific data have been hampered by the  lack  of

sufficient  data  storage  capacity to  accommodate  large  image

arrays.  Most  planetary images  require  nearly  a  megabyte  of

storage. Over the past two years a new storage technology "CDROM"

has been developed which provides the capability to store  up  to

600  megabytes of data on a single 4.72-inch disk. The disks  can

be  mass  produced,  and  hundreds of  copies  of  large  digital

archives distributed rather than the few sets currently produced.

Readers for CDROM disks can now be purchased for under a thousand

dollars   for  use  on  personal  computers  and   some   science

workstations.

The  PDS  has  worked with  industry  and  other  government

agencies  to  support the development of CDROM as  a  major  data

distribution  and storage media for space science  data.  Initial

development  efforts by the PDS, the Voyager Project, and  multi-

disciplinary   science  representatives  have  resulted  in   the

production  of 2 CDROM disks, one containing 800  digital  images

from   the  Voyager  Uranus  encounter,  and  one  containing   a

collection  of more than 1,400 data files from   all   scientific

disciplines.  The IMDISP program can be used to display  most  of

these  data  files on a personal computer equipped with  a  CDROM

drive.

2.1 DIGITAL IMAGE PROCESSING SUMMARY

A digital image is a picture converted to numerical form  so

that  it  can  be stored and used in a  computer.  The  image  is

divided  into a matrix of small regions called picture  elements,

or pixels. The rows and columns of pixels are called "lines"  and

"samples", respectively. Each pixel has a numerical value, or  DN

(data number) value, quantifying the darkness or brightness of

the image at that spot. In total, each pixel has an address (line

number,  sample  number) and a DN value, which is all  that  the

computer needs for processing. The DN value of each pixel usually

represents  a shade of darkness or brightness between  black  and

white  (gray levels). How many gray levels there are in an  image

depends  on  the  number of bits used  to  represent  each  pixel

intensity  in  the computer. The number of gray  levels  will  be

equal to 2^n, where n is the number of bits per pixel's DN value.

If 8 bits are used to represent a pixel's DN value (gray  level),

the  system will be capable of using 2^8, or 256, gray levels  in

an image, where DN 0 is pure black, and DN 255 is pure white. If

each DN used only 4 bits of storage, the image would contain only

2^4 or 16 gray levels; if there were only 1 bit per DN, the image

would contain only black and white pixels (bit values of 0 or 1).

2.2 OVERVIEW OF IMDISP CAPABILITIES

While  the  specifics of IMDISP commands  are  discussed  in

Chapter 4, this section provides a general overview of how a user

can manipulate images once a file is selected. There are  DISPLAY

commands which allow the user to display all or part of an  image

at  various  positions on the display screen. The user  may  also

zoom in and out from a point on the image defined by the  cursor,

and may pan around the image. The cursor may be turned on or off;

when  on, the arrow keys are used to move the cursor  around  the

screen; when off, the cursor is not visible.

A  user may choose to "subsample" the image, to enable  more

or  all  of the original image to be displayed on the  screen  at

once. For example, if the image were subsampled by a factor of 2,

every  other  pixel  from every other line  would  be  displayed,

starting from the upper left corner of the image. If a factor  of

3  were chosen, every third pixel from every third line would  be

displayed. Any positive integer may be used for subsampling.

The user may produce a histogram of an image file, which  is

a  graph showing the number of pixels per DN value, or per  range

of  DN values, for the entire image. Histograms are  overlaid  on

top  of the image, but may be removed using the REFRESH  command.

Subsampling may be specified when computing a histogram to reduce

the time required for the calculations.

A  profile may be created which plots DN value versus  pixel

along a line between two points on the image. After the cursor is

used to select the two endpoints, the connecting line is drawn on

the image and the profile is plotted at the bottom of the screen.

The user may also "stretch" the image, which is analogous to

turning the contrast knob on a TV set. The user specifies low and

high  DN values; all pixels with values lower than the  specified

"low"  become  black, and all pixels higher  than  the  specified

"high"  value become white. All pixels between the low  and  high

values are evenly shaded between black and white.

There  are bookkeeping types of commands, also. These  allow

the  user to do such things as retrieve a desired image from  the

CD-ROM,  to  save an image to a file on hard disk, and  to  erase

only the graphics from the screen or to erase the entire  screen.

Users  may  execute batch command files to do a series  of  tasks

automatically.  Lastly, the user may, of course, EXIT the  IMDISP

program when done.

2.3 GRAY LEVEL AND FALSE COLOR IMAGES

Most  planetary  images  are composed  of  8-bit  DN  values

representing monochrome brightness levels in the scene. To obtain

color  images,  separate images are taken through  color  filters

(red,  green,  blue) and are then combined by  ground  processing

systems to produce a true color image. Only a very small fraction

of planetary images are available in color versions. Most display

and analysis is done on monochrome images.

A display with 256 (2^8) gray levels is required to  present

the information contained in a standard image. However, the human

eye  can  only distinguish about 32 gray levels.  Thus  5-bit  DN

values  would satisfy most display  requirements.  Unfortunately,

computers  are  oriented to the storage of  and  manipulation  of

items  which are a power of two, and 5-bit pixels would  be  very

clumsy. The display devices which the IMDISP program supports are

limited  to 16 gray levels (PGA), 4 gray levels (EGA) or  2  gray

levels  (CGA).  The  16 gray levels of the PGA  are  adequate  to

support   image  analysis;  however  4  or  2  gray  levels   are

practically useless for viewing planetary images. Fortunately the

EGA display will support 16 different colors, and a color palette

can  be  selected  which  uses a graduated  scale  of  colors  to

represent  gray  levels, producing a "false color"  image  (false

because  the displayed color does not represent the actual  color

of the scene).

A  false color image (also called a pseudo color  image)  is

created from a black and white image by assigning a color (rather

than a gray level) to each DN value in the image. For instance, a

DN  value  of 128 could be reassigned to yellow if  the  user  so

desired.  Ranges of DNs (e.g., 100-125) may also be assigned  one

color. Pseudo colors get assigned to DN values in a pseudo  color

table,  which the display program then uses to determine  how  to

color  the  image  on the monitor. Display  programs  often  have

preset   pseudo   color  tables  with  commonly   used   DN-color

combinations, and the user can simply call for one of these  when

generating  a pseudo color image. The option exists,  of  course,

for  users  to  generate their own pseudo  color  tables.  Pseudo

colors are often used to highlight features of an unusual  nature

in an image.

Since  the Enhanced Graphic Adapter for the IBM PC  provides

only 4 gray shades (black, dark gray, light gray and white)  most

image viewing is done using a pseudo color table which interprets

gray  levels  as color values ranging from  black  through  reds,

greens and blues up to white.

2.4 RADIOMETRIC AND GEOMETRIC CORRECTIONS

All  camera systems have some kind of distortions  in  the

lens,  in  the  way the shutter works, etc.  Distortions  in  the

amount  of  fight transferred through the camera to  the  imaging

plate  are called radiometric distortions.  Geometric  distortion

affects  the "squareness" of an image, and is detected by  taking

an image of a geometric grid.

For  spacecraft  cameras, both types of  distortion  can  be

measured  before  launch, and in a limited  fashion,  in  flight.

These measured distortions are saved in digital files. When a raw

image is received, these "calibration files," as they are called,

are  subtracted  from  (or  otherwise  applied  to)  the   image,

effectively  removing radiometric and geometric distortion.  This

process is called radiometric and geometric correction. Corrected

images  contain the closest possible representation of the  scene

being imaged.

2.5 IMAGE FILE FORMATS

Digital  image files are seldom stored as a simple array  of

pixels.  Generally there is an area at the beginning of an  image

file containing descriptive information about the image. This  is

referred to as a label or header area. Figure 2-1 shows a diagram

of a simple image format with a label area followed by the  image

lines.  In practice, images from planetary missions have  a  more

complicated   format,  due  to  the  need  to  store   additional

information  to  allow proper interpretation of the  image  data.

Figure  2-2  shows  the  format of a  Voyager  image,  which  has

engineering parameters embedded at the end of each image line  to

form a line suffix area, an engineering trailer record after  the

last image line, and this is followed by an image histogram.

--------------------------

|  Labels or Header Area   |

|--------------------------|

|  Image Array Line 1      |

|--------------------------|

|  Image Array Line 2      |

|--------------------------|

.

.

.

--------------------------

|  Image Array Line n      |

--------------------------

Figure 2-1: Simple Image Format

Different image label formats have been developed for use by

nearly  all image processing facilities. Two formats  are  widely

used  within the planetary and astronomy communities: the  VICAR2

(Video  Image  Communication and Retrieval) labels  used  by  the

Multi-Mission   Image  Processing  Laboratory  (MIPL)  and   FITS

(Flexible  Image  Transport)  labels  used  for  astronomy  image

interchange.  The PDS has developed a label scheme which is  very

similar  to  these  standards,  called  the  Object   Description

Language  (ODL). This format attempts to add a broader  range  of

data  descriptive capabilities to the proven capabilities of  the

existing label systems. A description of the ODL architecture  is

given in Appendix D.

Most of the images stored on the PDS CDROM disks have either

ODL  or VICAR2 labels. IMDISP will automatically interpret  these

labels to determine the display format of the image data.

--------------------------------------

|  PDS ODL Labels                      |

|--------------------------------------|

|  Image Array Line 1    | Line Suffix |

|--------------------------------------|

|  Image Array Line 2    | Line Suffix |

|--------------------------------------|

.

.

.

---------------------------------------

|  Image Array Line 800  | Line Suffix |

|--------------------------------------|

|  Image Engineering Trailer           |

|--------------------------------------|

|  Image Histogram                     |

--------------------------------------

Figure 2-2: Voyager CDROM Image Format

2.6 CDROM FUNDAMENTALS

The CDROM disk uses the same basic data storage format as  a

CD  audio  disk. In fact, data blocks are identified  by  minute,

second and sector number, following the audio format. Data blocks

are recorded along a spiral from the inner to the outer radius of

the  disk. Each raw data block (sector) contains 2,352  bytes  of

information,  with  304  bytes used for  housekeeping  and  error

correction  and 2,048 (2K) containing user data. Each data  block

is  called a sector, and 75 sectors are stored per second.  Since

the  nominal  playing time of a CD disk is 60 minutes,  the  data

storage capacity is 75 sectors per second * 60 seconds per minute

*  60  minutes  or  270,000 sectors.  Thus  the  nominal  storage

capacity  of a single CDROM disk is 540,000 kilobytes, which  can

be  extended  to  more  than  600,000  kilobytes  and  beyond  by

recording more than 60 minutes of data on a disk.

In  order  to maximize the storage capacity of  CD  disks  a

constant  linear  velocity (CLV) recording format is  used.  This

means that the player changes speed (slows down) as it reads from

the  inside  to the outside of the disk, to maintain  a  constant

flow  of data under the read mechanism at a speed of  1.2  meters

per  second.  This is in contrast to most  magnetic  disk  drives

which  use  constant angular velocity (CAV)  storage,  where  the

information density is greater on the inner tracks that the outer

tracks.  While  the use of CLV recording  increases  the  storage

capacity  of CDROM disks, it reduces access time, since the  disk

must change speeds as it moves to different positions on the disk

radius.  The strategy for locating a recorded data block is  also

more  complicated than with constant angular  velocity  recording

technology.

As  a  result  the average access time of  CDROM  drives  is

between 400 ms and 1 s, and the maximum data transfer rate to the

host  computer  is  150 kilobytes per  second.  These  rates  are

approximately  an order of magnitude slower than magnetic  disks.

Therefore,  the  access mechanisms to disk directories  and  data

must  be  customized  for CDROM in order  to  provide  acceptable

performance.

The  development of a standard logical format for  recording

data  files  on  CDROM disks was taken on by  a  group  of  CDROM

applications  developers, hardware vendors and computer  vendors.

The  proposed standard resulting from this effort is  called  the

High   Sierra   format.  This  format  was   submitted   to   the

International Standards Organization (ISO) for consideration  and

approved  on October 5, 1987. The format used for the  PDS  CDROM

disks is dated May 28, 1986, and differs slightly from the  final

ISO standard. Microsoft and other CDROM software developers  have

committed to supporting this preliminary version of the format in

later  software  releases. However, software developed  by  other

vendors  to  support the ISO format (DEC for  example)  will  not

support the format of these disks.

Chapter 3

IMDISP INSTALLATION

IMDISP requires an IBM PC, XT, AT or 100 percent  compatible

with  512K of base memory. It supports several  graphics  display

devices,  including  the  Enhanced Graphics  Adapter  (EGA),  the

Video Graphics  Array  (VGA), the Professional  Graphics  Adapter

(PGA),  and  the  Color  Graphics  Adapter  (CGA).  The  hardware

environment  in which  the program  has been developed and tested

includes and  AT class computer,  deluxe or  enhanced  EGA  board

with 256K of memory, and multisync or multiscan monitor.

3.1 PROGRAM SETUP

The IMDISP program can be run from a floppy or hard disk. To

use  the program from a floppy disk, insert the program  disk  in

the  A: or B: drive and type the MSDOS command: "A:" or  "B:"  to

set the floppy disk as the default drive. To use the program from

hard disk first create a directory to hold the IMDISP files.  Use

the  MSDOS command "MKDIR \IMDISP" to create a directory to  hold

the programs. Use the "CHDIR \IMDISP" command to make IMDISP  the

default directory. Insert the distribution floppy disk in  floppy

disk  drive A:. Now type "COPY A:*.*" to copy the files from  the

distribution  disk to your hard disk. You should now be  able  to

run  all examples from that directory on your hard disk.  If  you

wish  to use IMDISP from other directories, you need  to  include

the  IMDISP directory in your MSDOS "PATH" command. This  command

provides MSDOS with a list of directories to search when  looking

for  a  program  file  to run. If you wish  to  run  IMDISP  from

different  directories, the \IMDISP subdirectory should be  added

to  the  PATH command in your AUTOEXEC.BAT file.  A  sample  path

command would look like this:

PATH=C:\DOS;C:\UTIL;C:\WORDSTAR;C:\IMDISP

The  sample  batch  files and special  pointer  label  files

included  on the distribution disk will only run if your  current

default directory is \IMDISP.

You  may want to add a line to your AUTOEXEC.BAT file  which

will  allow you to abort the display program while running  batch

files or displaying large image files. If the command  "BREAK=ON"

is  placed  in  CONFIG.SYS,  you  will  (sometimes)  be  able  to

terminate the IMDISP program by typing the control and break keys

simultaneously. Different computer systems offer varying response

to  the  control-break command, and the use of this  command  may

effect other programs you use.

3.2 CDROM SETUP

To use the IMDISP program with PDS CDROM disks you will need

a  CDROM  reader, interface board, and software provided  by  the

vendor  which will make your CD Reader look like a disk drive  to

your  PC.  The details of setting up your hardware  and  software

configuration  are  beyond  the scope  of  this  manual,  however

several aspects of setup will be reviewed.

There  are  many  potential  pitfalls  in  setting  up  your

hardware and interface board. Read the directions carefully,  and

don't  assume that "default" switch settings are  correct.  Check

all  switches  to  see that they correspond  to  the  recommended

settings.

To  work  properly with the batch command  and  label  files

provided  with  IMDISP, your CDROM drive should be set up  to  be

drive letter "L:". Most vendor software allows you to select  the

drive letter which will be assigned to the CDROM reader.

3.2.1 Microsoft Extensions

If  you are using the Microsoft Extensions you will  load  a

CDROM  device  driver  as part of your CONFIG.SYS  file  (or  two

drivers  if  using the Philips or DEC readers) with a  line  like

this:

DEVICE=HITACHI.SYS /D:CDROM1 /N:1

This command assumes that the driver file "HITACHI.SYS"is located

in  your ROOT directory. The /D:CDROM1 switch assigns  a  logical

name CDROM1 to the drive. A name must be supplied, and is used to

identify this device, in case more than one CDROM drive is  being

used on your system. The name should not be the same as the  name

of any file stored on your system. An attempt to open a file with

the  same  name as the device driver will result  in  the  device

driver being opened, not the file. The /N:1 switch indicates that

this  is drive 1. This switch is used since some CDROM  interface

cards support multiple drives.

Other commands which should be included in CONFIG.SYS if you

are using a CDROM drive with the Microsoft Extensions are:

LASTDRIVE=Z

FILES=30

BUFFERS=50

The LASTDRIVE command tells MSDOS to allow for enough  drive

letters to support the CDROM drive, plus any other drives on your

system. Using LASTDRIVE=L should also work in most circumstances.

The  files and buffers specifications provide  sufficient  system

work area to support CDROM operations.

You  must  also execute a program  called  MSCDEX.EXE  which

allows  access to the CDROM drive as if it were a disk  drive  on

your system. This command can be put in your AUTOEXEC.BAT file so

that it is automatically run whenever you start-up your computer.

The command format is:

MSCDEX /D:CDROM1 /L:L /M:20 /V /E

Several  MSCDEX command switches are illustrated.  The  /L:L

command  assigns  the drive name L: to the CDROM  drive.  The  /D

switch  gives the device name of the CDROM drive (which  must  be

the  same  name  used  in  the  device  name  parameter  of   the

"DEVICE=CDROM.SYS"  command  in the CONFIG.SYS  file.  The  /M:20

switch  assigns  twenty 2K blocks of memory as a buffer  for  use

with CDROM data. The /V switch provides verbose messages when the

installation  program  is run, and /E tells the  program  to  use

expanded memory for the cache area.

3.2.2 Other Driver Software

Some   vendors  (Reference  Technology,  TMS)  offer   CDROM

software  which  is comparable to the Microsoft  Extensions.  Our

experience  with these software packages is that they modify  the

internals of MSDOS and may cause unpredictable results with  your

other  PC software. They often do not support CDROM  applications

designed  to work with the MSDOS extensions for CDROM and  cannot

be used with the IMDISP FILE prompt mode.

3.2.3 Problems with CDROM Software

The  PDS CDROM disks utilize a feature of the  CDROM  format

standard which provides extended attribute records to define  the

physical  characteristics  of data files. These records  will  be

used  by VAX and other minicomputer systems where  the  operating

system  allows  a  variety of record  formats  (fixed,  variable,

stream).  Many  of  the earlier implementations  of  High  Sierra

software  failed to recognize these records, which are placed  at

the  beginning of a file's data area. If your CDROM software  was

developed prior to April 1987, it may interpret these records  as

part of the data file, causing the first 2K bytes of each file to

appear as meaningless binary data. Users with this problem should

contact  their hardware or software vendors for updated  versions

of  the  CDROM software. The IMDISP program has been  patched  to

recognize and skip over these records on the PDS CDROM disks, but

these  patches  may not support other CDROM disks  with  extended

attribute records.

3.3 DISPLAY DEVICE CHARACTERISTICS

IMDISP  automatically  finds out which display  devices  are

available  (PGA,  EGA, or CGA) and uses the  one  available.  The

Enhanced  Graphics Adapter must have at least 128K of  memory  to

work  properly. Use of the Color Graphics Adapter is  discouraged

because the pixels have no gray levels, only black or white.

The  display  coordinates start at (1,1) in the  upper  left

corner  of the screen; the line direction is down and the  sample

direction is to the right.

3.3.1 Enhanced Graphics Adapter (EGA)

The   EGA  is  a  bit-mapped  display  device  providing   a

resolution  of  350  lines  by 640 samples  of  4-bits  each.  It

produces  a  digital video signal for each  primary  color  (red,

green  and blue), The video signal for each color can be  set  to

one  of four levels, roughly equivalent to off, low,  medium  and

high.  Thus the number of possible colors (color palette)  is  64

(4^3). The following table illustrates the colors created by some

of the different combinations of red, green and blue.

Red=   off, Green=   off, Blue=   off   results in Black

Red=   low, Green=   off, Blue=   off   results in Dark Red

Red=medium, Green=   off, Blue=   off   results in Light Red

Red=  high, Green=   off, Blue=   off   results in Bright Red

Red=   low, Green=   low, Blue=   low   results in Dark Gray

Red=medium, Green=   low, Blue=   off   results in Brown

Red=medium, Green=medium, Blue=   off   results in Yellow

Red=medium, Green=medium, Blue=medium   results in Light Gray

Red=  high, Green=   low, Blue=   off   results in Orange

Red=  high, Green=  high, Blue=  high   results in White

Table 3-1: Sample EGA Color Palette Settings

The  EGA display is limited to only 16 colors out of the  64

possible, because only 4-bits are used to store each pixel  value

in memory. This 4-bit pixel value points to an entry in the color

table  which  represents one of the 64 possible  combinations  of

red, green and blue which can be displayed.

Many  of  the  newer EGA boards are  capable  of  displaying

additional lines and samples when using a Multisync or  Multiscan

monitor.  The  standard  EGA uses a  16  Mhz  crystal  oscillator

(something like a clock which regulates the display speed), which

limits  the video output rate to the equivalent of 640 pixels  by

350  lines.  The  EGA  circuitry  allows  for  a  faster  crystal

oscillator to be added to the board (24 Mhz and beyond)  allowing

about 25 percent more lines to be displayed (480 lines instead of

350).  A  method for adding this capability to  an  existing  EGA

board  (for  about  $10  worth of  parts)  is  described  in  the

September 16, 1986 issue of PC Magazine.

This mode requires that a multisync or multiscan monitor  be

attached to the computer, and there is no way for the program  to

automatically  detect  the  presence  of  the  special   monitor.

Therefore  this  display mode is invoked by specifying  an  MSDOS

"ENVIRONMENT"  variable. This is done by issuing an  MSDOS  "SET"

command at the MSDOS prompt as follows:

SET EGA480=TRUE

This command can also be put in your AUTOEXEC.BAT file using

a  text editor, so that you need not invoke it each time you  run

IMDISP.   It  should  have  no  effect  on  other  MSDOS   system

operations.  There  is a chance that adding  this  variable  will

exceed  the size reserved by MSDOS for environment variables.  If

so,  you  should  consult  your  MSDOS  manual  to  increase  the

environment size on your system. The environment variable can  be

removed with the MSDOS command:

SET EGA480=

Where a carriage return is typed immediately after the equal

sign. The 480 line mode requires an additional 83K of memory  for

the refresh buffer. If you try this mode and receive the  message

"Insufficient memory for line buffer", it is probably because you

have  a  512K memory machine, or have  memory  resident  programs

operating (like Sidekick or Superkey) which reduce the  available

memory to less than about 400K. You will need to use the 350 line

mode  or remove some memory resident programs to operate  in  480

line mode.

Users  of the EGA with Multisync and Multiscan monitors  can

achieve  a  16 gray level display through the use  of  a  special

device  called the Grayscaler 1, from Avocado Computer, Box  632,

Yorba  Linda, Ca, 92686, (714) 528-1025. This cable converts  the

digital  video  signals  produced  by the  EGA  board  to  analog

signals.  By selecting appropriate EGA palette entries, a  fairly

good  representation of 16 gray levels can be achieved  (use  the

palette selection "PAL LOAD GRAY16.PAL" to select this palette).

3.3.2 Video Graphics Array (VGA)

This version (2.1) of the IMDISP program includes support for the

IBM Video Graphics Array (VGA) display device.  The VGA  produces

an  analog display with several new color modes.   Of  particular

interest  to  IMDISP users are the 640 x 480 line  mode  with  16

displayable  colors (or gray levels) and the 320 x 240 line  mode

with 256 displayable colors (or 64 gray levels).  This version of

IMDISP supports the 640 x 480 mode.

To  set  the VGA display mode you must set  a  MSDOS  environment

variable prior to executing IMDISP.

To set the environment variable use the MSDOS command:

SET VGA=TRUE

This will put the display in 640 sample by 480 line mode, with 16

gray  levels or colors displayable from a palette of  256K.   The

display  is  initialized  with  a  gray  scale  palette,  but   a

pseudocolor  palette can be invoked with the IMDISP command  "PAL

PS 0".  The "PALETTE EDIT" command will allow you to step forward

or  backward  through the 64 available shades  for  each  primary

color.   See  the  next  section  of  the  IMDISP manual for more

information on the palette edit function.

3.3.3 Professional Graphics Adapter (PGA)

The PGA is a special graphics board developed by IBM for use

with CAD/CAM applications on the IBM PC. Because its architecture

is incompatible with the CGA and EGA boards, it has not been well

received by software developers. The PGA provides an analog video

signal  with a display resolution of 480 lines by 640 samples  of

8-bits  each. It provides 16 intensities for each primary  color,

resulting  in a palette of 4,096 colors (16^3). The  color  table

allows 256 colors to be displayed simultaneously, however only 16

gray levels can be selected.

3.3.4 Color Graphics Adapter (CGA)

The  CGA  display  is a bit-mapped graphics  device  with  a

resolution of 200 lines by 640 samples of 1-bit each. It supports

only 2 colors in this display mode, black and white. This display

can  be used to get a general idea of the contents of  a  digital

image,  or for displaying one bit images (graphics  for  example)

but is not recommended for use with the PDS CDROM images.

Chapter 4

IMDISP OPERATION

IMDISP can be used to display images up to several  thousand

lines and samples with a variety of pixel formats. These  formats

include  bit (1 bit), nibble (4 bits), byte (8 bits), or  integer

(16  bits). Integer pixels are assumed to be byte-swapped,  which

means  that  the  sign and most significant 7 bits  are  in  the

rightmost  byte,  and  the least significant 8 bits  are  in  the

leftmost byte. This is the convention used by both the IBM PC and

VAX  computer hardware families. IBM mainframes and 68000  series

computers  (Macintosh and Sun, for example)  use  un-byte-swapped

integers.

Program  interaction with the user is via commands typed  in

response to the "COMMAND:" prompt. Most interaction is  performed

on  the graphic display screen, and program messages are  printed

over  any  image  display currently on the  screen.  The  REFRESH

command can be used to restore an image after other commands have

written  text over the display. The ERASE command can be used  to

clear  away  the  clutter left by  program  status  messages  and

previously  displayed  images. The screen  is  not  automatically

erased after each operation in order that multiple images can  be

displayed simultaneously, or so a histogram can be placed on  the

screen with the image.

IMDISP  is invoked by typing "IMDISP" or "IMDISP  filename",

where  filename  is the name of an image file to  be  opened  for

processing  at program start-up. The filename may include a  disk

drive and path name specification.

The  program  will blank the display screen  and  display  a

welcome logo, then the prompt "COMMAND:" will appear in the lower

left  portion  of the screen. If a filename is  included  in  the

command invocation, the welcome message is not displayed and  the

screen will display the COMMAND: prompt.

Typing  "HELP" at the command prompt will provide a list  of

IMDISP commands. The most frequently used commands are "FILE"  to

open  a  file for processing, and "DISPLAY" to display  an  image

once the file has been opened.

4.1 COMMAND SYNTAX

The command line syntax is of the form:

COMMAND KEYWORD1 = VALUE1 KEYWORD2=VALUE2 KEYWORD3 VALUE3 . . .

The command line may be typed in either upper or lower case.

The  command  and  the  keyword names may  be  abbreviated  to  3

characters  in most cases. Any number of spaces may  be  inserted

between  words, and the keyword and value may be separated  by  a

space  or an equals sign. Some keywords do not require  a  value.

All  keywords are optional and have default values.

4.2  COMMAND DESCRIPTION

Table  4-1 presents a summary of IMDISP commands.  They  are

separated  into  3 groups, file  manipulation  commands;  display

commands  and  program  control commands.  Only  the  capitalized

letters are required to specify a command.

4.3 OPERATING MODES

Several  of  the commands invoke special  program  operating

modes.  These  include  the FILE command when  issued  without  a

filename  argument  and  the CURSOR,  PROFILE  and  PALETTE  EDIT

commands.  The FILE mode displays a list of files in the  current

directory  on the screen for selection. It also contains  several

subcommands  for  controlling the display of files  on  the  menu

screen.  In  the CURSOR, PROFILE and PALETTE  EDIT  commands  the

cursor  keys  (arrow  keys) on the numeric  keypad  are  used  in

conjunction  with other keys to control program operation.  These

modes  are  exited  by typing either a  period  '.'  or  carriage

return.

File oriented commands:

CD or CHDIR      . . . . to change the default directory

DIRECTORY        . . . . to perform the MSDOS directory command

FILE             . . . . to specify the name of the image

SAVE             . . . . to save the image display to a file

TYPE             . . . . to perform the MSDOS type command

LABEL            . . . . to display the image labels

Display commands:

DISPLAY          . . . . to display the image

ERASE            . . . . to erase the display

REFRESH          . . . . to refresh the image plane

PALETTE          . . . . to adjust the palette for the display

HISTOGRAM        . . . . to display the histogram of the image

SET              . . . . to set DN value range

STRETCH          . . . . to do a linear gray scale stretch

CURSOR           . . . . to move the cursor around

PROFILE          . . . . to plot a profile of the image

TEXT             . . . . to draw text on the image

Program control commands:

BATCH            . . . . to execute a batch command file

MENU             . . . . to select images from a menu file

EXIT or QUIT     . . . . to exit from the program

HELP             . . . . Display help information

SYSTEM           . . . . execute an MSDOS command

Table 4-1: IMDISP Command Summary

4.4 ERROR HANDLING

If  an invalid command is issued at the COMMAND: prompt  the

program will beep and return to the COMMAND: prompt. In  handling

command  parameters the program takes action on those  parameters

that are recognized, but ignores invalid parameters. For example,

issuing a "SET LO = 25 HI = 100" command will not have any effect

since  the proper parameter names for the SET command are  "DNLO"

and "DNHI".

It  is  also  possible  for  certain  system  errors  to  be

encountered  which  cause  the  program  to  abort  leaving  your

computer  in  graphics mode. N you are  using  certain  utilities

which reset the default text mode screen colors (like the  Norton

Utilities  screen attributes command) you may not be able to  see

what is being printed on the screen. The simplest approach is  to

perform  a  warm  boot,  by  typing  the  Ctrl,  Alt,  Del   keys

simultaneously. Alternately, you can often type IMDISP then  EXIT

to  reset the computer to text mode. Errors in  processing  image

files on CDROM, hard disk or floppy disks can cause these aborts.

4.5 COMMAND REFERENCE

This  section lists all IMDISP commands  alphabetically  and

describes  their  function,  parameters  which  control   command

operation,  and provides examples of command use, In the  command

and parameter description the following conventions are used:

filename    represents the name of an MSDOS file, and may include

an optional drive identifier and path specification.

n           represents an integer value.

4.5.1 BATCH

The  BATCH command reads commands from the  specified  batch

command file. If no file name is specified the current  directory

is  searched for a file named BATCH.CMD. The CURSOR, PROFILE  and

PALETTE  EDIT  commands should not be used within  batch  command

files since they require interactive inputs from the keyboard.

The BATCH command may take the following argument:

filename    filename of batch command file.

A  batch  file is simply a text file containing  a  list  of

commands,  with each command terminated by a carriage return.  If

the  command file is created with a word processor,  the  program

should be used in the non-document mode, to assure that  carriage

returns  are  embedded  between  command  lines.  An  example  of

creating a batch file to display the PDS logo image follows.

At the MSDOS command level type:

COPY CON TEST.CMD

FILE LOGO.IMG

DISP ZOOM 4

DISP ZOOM 2

DISP

DISP SUB 2

EXIT

Now  run IMDISP and type"BATCH TEST.CMD" at  the  "COMMAND:"

prompt.  Make  sure  the  file  "LOGO.IMG"  is  in  your  current

directory.  The  PDS logo should be displayed on  the  screen  in

several different sizes.

4.5.2 CD or CHDIR

The  CD or CHDIR command is used just as it is at the  MSDOS

command level to change the current default directory.

The   argument   of  the  CD  command  is  the   path   name

specification  of  the new default directory.  For  example,  "CD

\IMDISP\IMAGES"  would make \IMDISP\IMAGES the current  directory

if  it  exists.  To  change the current  default  drive  use  the

"SYSTEM"  command  ("SYSTEM  L:"   for  example)  or  the  "FILE"

command.  If there is not enough memory available for the  system

command to be executed an error message is displayed.

4.5.3 CURSOR

The CURSOR command is used to select a point in a  displayed

image  for  subsequent display commands. When first  invoked,  it

places  a  small  cursor  symbol at the  center  of  the  screen.

Depending  on the current contents of the screen, the cursor  may

be  hard  to see. Moving it with the arrow keys  on  the  numeric

keypad will help you locate it.

Whenever  the cursor is moved, the current line  and  sample

location  of  the cursor and the DN value of the  pixel  at  that

point  is printed in the lower right portion of the  screen.  The

cursor  will move 16 pixels in the selected direction unless  the

movement value is adjusted using the + or  keys.

Numeric Keypad Keys

-------------------------------

|   7   |   8   |   9   | PrtSc |

|  Home |  Up   |  PgUp |       |

-------------------------------

|   4   |   5   |   6   |   -   |

|  Left |       | Right |       |

-------------------------------

|   1   |   2   |   3   |   +   |

|  End  | Down  |  PgDn |       |

-------------------------------

Action of Keypad Keys:

7 = Move up and left.

8 = Move up.

9 = Move up and right.

4 = Move left.

5 = Not active.

6 = Move right.

1 = Move down and left.

2 = Move down.

3 = Move down and right.

- = Decrement cursor movement value.

+ = Increment cursor movement value.

Table 4-2: IMDISP Cursor Keys

Cursor  mode is exited by typing a "." (period) or  carriage

return. The cursor position is retained for future use in DISPLAY

commands  which  use the CENTER option. It is  very  useful  with

large  images where the entire image may be displayed  using  the

subsample option, then a point selected for subsequent display at

full resolution.

Example:  Type  "FILE  MONTAGE.IMG" followed  by  "DISP"  to

display  the  MONTAGE.IMG file.  Type "CUR" to enter  the  cursor

mode,  then move the cursor to the center of the Uranus image  in

the  upper left corner of the image using the up and  left  arrow

keys.  Now type "." to exit cursor mode, then type "DISP  ZOOM  4

CENTER"  which will display Uranus as a large grainy ball on  the

screen.

4.5.4 DIR

The  DIR command is used just as the MSDOS dir command.  Its

argument  is  passed to MSDOS for execution. At the  end  of  the

directory  listing the screen will display "Type carriage  return

to  continue:" which will return to the IMDISP command  mode.  If

there is not enough memory available for the system command to be

executed an error message is displayed.

4.5.5 DISPLAY

The  DISPLAY command reads an image file and displays it  on

the  the screen. If the image is larger than the display  screen,

the default mode is to display as much of the upper left  portion

of the image as will fit on the screen. The SUBSAMPLE keyword  can

be   used  to  select  every  'n'  lines  and  samples  so   that

representations  of very large images may be displayed. The  ZOOM

keyword  can  be  used to enlarge a portion of an  image  on  the

display screen. Keywords can be specified to begin displaying  at

any line and sample in the image with the SL (starting line)  and

SS  (starting  sample).  If only a limited number  of  lines  and

samples  are  to  be displayed the NL (number of  lines)  and  NS

(number of samples) keywords can be used. The UP, DOWN, LEFT  and

RIGHT  keywords can be used to display different portions of  the

image,  relative to the currently displayed portion. The  display

can also placed at any point on the screen using the DSL (display

starting line) and DSS (display starting sample) keywords.

DISPLAY takes the following keywords :

SUBsample n  a positive integral subsampling factor for

displaying images larger than the size of the

screen.  SUB=2 would display an image at half

the resolution.

If the image lines and samples look like this:

sample values

line 1            1  2  3  4  5

line 2            6  7  8  9 10

line 3           11 12 13 14 15

Then the command DISP SUB 2 would produce:

sample values

line 1            1  3  5

line 2           11 13 15

ZOOM n        a positive integral zoom factor for looking at

part of an image close up.  The zoom command

replicates pixels by the integer value specified.

If the image lines and samples look like this:

sample values

line 1            1  2  3

line 2            4  5  6

line 3            7  8  9

Then the command ``zoom 2" would produce the following:

sample values

line 1            1  1  2  2  3  3

line 2            1  1  2  2  3  3

line 3            4  4  5  5  6  6

line 4            4  4  5  5  6  6

line 5            7  7  8  8  9  9

line 6            7  7  8  8  9  9

CENTER     center the display around current cursor position.

This command is very useful when displaying large

images or when zooming in on selected areas.

NL n

NS n       number of lines or samples from the image file to display

SL n

SS n       starting line or sample within the image file to display

UP n

DOWN n     adjusts the starting line up or down from the

previous value, thereby moving the display window

up or down in the image.  The default amount to

move up or down is the size of the display.

LEFT n

RIGHT n    adjusts the starting sample left or right from the

previous value, thereby moving the display window

left or right in the image

DSL n

DSS n      start the image display at the specified line and

sample on the display screen.

Examples: The user wants to display an entire Voyager  image

(800  x 800) on the display screen then display a portion of  the

image  at full resolution. Type "FILE MIRANDA1.LBL",  then  "DISP

SUB  2" to display the entire image at half resolution,  Use  the

CURSOR  command  to move the cursor to the center of an  area  of

interest.  Exit  the cursor mode by typing ".", then  type  "DISP

CENT"  to  display  the selected portion of  the  image  at  full

resolution.

The  user wishes to place 2 images which are each 800  lines

by  800 samples on the screen next to each other. The first  file

is opened with "FILE IMAGE1.IMG".  The "DISPLAY SUB 4" command is

given to display every 4th line and sample of the image to create

a 200 x 200 display. Next the "FILE IMAGE2.IMG" command is given.

Now a "DISPLAY SUB 4 DSS 201" is given to place the second  image

on  the screen starting at display sample position 201,  next  to

the first image.

4.5.6 ERASE

The command ERASE causes the screen to be erased by  setting

all  of  the  pixels to 0, which may  not  necessarily  be  black

depending on the palette setting.

4.5.7 EXIT

The  EXIT command exits IMDISP and returns the user  to  the

MSDOS  operating  system. The contents of the image  display  and

refresh buffer are discarded.

4.5.8 FILE

FILE does not take a keyword, just a value which is the file

name.  The  specified file will be opened, or  an  error  message

issued  if the file cannot be opened. The capability to  directly

address  any sector on the CDROM disk by providing  its  physical

address  (minute, second and sector number) is also available  to

users of the Microsoft Extensions software.

If  the  FILE command is specified without a  filename,  the

program  will  display  a  list of  file  names  in  the  current

directory.  Each  file name is preceded by a number.  Typing  the

number  associated  with a file name will select  that  file  for

processing.  File  names  that  actually  represent  lower  level

directories  are  indicated  with a <d> symbol  after  the  name,

Directories  can  be traversed downward by selecting  the  number

associated  with  a  directory  name.  Upward  traversal  is   by

selecting  the  item  identified as PARENT DIR.  This  will  move

upward in the directory hierarchy.

The  prompt  mode clears the screen and displays a  menu  of

files  and commands. These commands allows specification  of  the

default  drive, a file "mask" to use in selecting file names  for

display and options for moving through the list of files when the

current directory contain more than 30 files. The top line of the

display  also  indicates  how  many  files  are  in  the  current

directory.  After  exiting  the FILE prompt  mode,  the  previous

contents  of  the  display  screen can  be  recovered  by  typing

"REFRESH".

The FILE prompt mode subcommands:

D       select default disk drive

M       specify a file selection mask to use in displaying

files on the screen.

N       display the next set of file names on the screen

if there are more files in the current directory

than can be displayed on one screen.

P       display the previous set of file names.

Q       quit the file selection mode and return to IMDISP

command mode.

The  N, P and Q commands are executed immediately. If the  D

or M commands are selected, the user is prompted to enter a drive

letter or file mask.

If  the file selected by the "FILE filename" or FILE  prompt

mode has a valid label, either PDS or VICAR2, then the number  of

lines  and samples and the pixel size will be displayed.  If  the

file  is unlabeled then the program will prompt for the values  to

use  for the number of lines and samples, the number of bits  per

pixel,  and  the  number of header bytes.  PDS  detached  labeled

images  can  also be displayed. The specified  file  remains  the

current file until a new FILE command is given. The FILE  command

resets  the  SET  DNLO and SET DNHI values  to  the  minimum  and

maximum  for the pixel size (normally 0 and 255). If a  specified

filename  is  not found in the current directory  then  an  error

message is displayed.

If  the  unlabeled  image has a regular format,  that  is  a

uniform  structure of repeating lines and samples, the  user  can

specify  this  information  when prompted  by  the  program.  For

example,  an  image named ASTERIA.DAT with one 1422  byte  header

block  followed by an image composed of 370 lines by 450  samples

of 8 bit pixels would be processed as follows:

At  the COMMAND: prompt type "FILE ASTERIA.DAT" The program  will

display the message:

Input file does not have a proper label.

Input number of lines: 370                enter the value 370

Input number of samples: 450              enter the value 450

Input size of each sample: 8              enter the value 8

Input size of header label: 1422          enter 1422 for the

size of the foreign

label block).

The  COPIM  utility can be used to add a PDS  label  to  the

beginning  of  such a file, so that it can be  used  subsequently

without having to specify all the parameters separately.

The  physical address of a data block on the CDROM can  also

be  specified using the syntax 'FILE "CD:mm:ss:bb"', where mm  is

the minute, ss the second, and bb the sector block number of  the

beginning  of the desired data area. The CD:mm:ss:nn must  be  in

double quotes and the CD must be capitalized! This mechanism  can

be  used  to access files on non-High Sierra format  disks.  This

will  only  work  on  systems  using  the  Microsoft   Extensions

software.

4.5.9 HELP

HELP  displays the online help message. If followed  by  the

name  of  a  valid  command  the  message  for  that  command  is

displayed.  HELP is not provided for MSDOS commands (DIR,  CHDIR,

CD and TYPE).

4.5.10 HISTOGRAM

The HISTOGRAM command produces a plot showing the number  of

pixel  values  in  the image at each DN level. The  range  of  DN

values  is displayed on the x-axis and the number of  occurrences

of a specific DN value on the y-axis (See Figure 4-1).

The  HISTOGRAM command reads through the  image,  calculates

the  histogram,  and then displays the plot on  the  screen.  The

histogram is scaled so that the third highest histogram value  is

used  as  the maximum. It is recommended that  the  SUBSAMPLE  or

other   subsetting  keywords  be  used  to  speed  up   histogram

calculation,  which  can be quite slow for large images  and  for

images  stored on CDROM. Histogram plots are displayed on top  of

any image currently displayed on the screen, so the user may wish

to  use  the  ERASE command to clear the screen  prior  to  using

HISTO.  After  the  histogram  is displayed,  the  image  can  be

restored using the REFRESH command.

27582  |            .

|           ..

|          ....

COUNT     |         .....

|        .......

|       .........

|     ............

|   ...............

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

0      64      128     192     256

DN value

Figure 4-1: Sample Image Histogram

By  evaluating  the histogram, the user  can  determine  the

optimum settings for the SET (DNLO and DNHI) command to produce a

useful  display.  In  the  example  above,  the  image  data  are

clustered  between  DN  values  32 and  144.  Using  the  default

settings  of  the EGA display mode the colors assigned to  the  2

lowest  color  values and the 7 highest values  (representing  DN

values from 0 to 31 and from 144 to 255) would not be used  since

the image contains no pixels with these values.

By  using the "SET DNLO 32 DNHI 144" command the 16  display

levels  available on the EGA display would be assigned to  values

between 32 and 144 rather than distributed evenly from 0 to  255,

and all 16 colors would be used in the display.

HISTOGRAM takes the following keywords and arguments:

SUBSAMPLE  n   a integral subsample  factor

for speeding up histogram use of

"HISTOGRAM SUBSAMPLE 4" is always recommended

NL n

NS n           number of lines and samples from image

SL n

SS n           starting line and sample in image

Two  special  keywords can be used with  Voyager  or  Viking

images  on  the PDS CDROM disks.  These  will  extract  histogram

values  stored  with the image on the CDROM disk and  reduce  the

time required for the histogram to be generated from more than  1

minute to about 3 seconds. Note that these commands will not work

with Voyager image files in the \PLANET\JUPITER\ATMOS  directory,

or with versions of system software written before April 1987.

VOY     use histogram from Voyager CDROM engineering data

VIK     use histogram from Viking CDROM engineering data

4.5.11 LABEL

The label command prints the labels for the currently selected

image file on the screen.

4.5.12 MENU

The menu command reads a file containing a list of file names and

places the user in the MENU mode, where files from this list can be

selected for display.  The MENU mode is similar to the FILE mode,

except that the list of files presented is that provided in the menu

file, rather than from a selected directory.

The MENU mode can be invoked by typing "MENU filename.MNU" at the

COMMAND: prompt.  It can also be invoked on program startup by typing

"IMDISP filename.MNU" to initiate the program.  The menu file must

use the ".MNU" extension to startup IMDISP in MENU mode.

Subcommands of the MENU mode are as follows:

# - Enter the number corresponding to a desired file name to select

the file.

P)revious  - display the previous screen of file names

N)ext      - display the next screen of file names

Q)uit      - quit or exit from MENU mode

After selecting and displaying an image, the user can return to the

current menu by typing "MENU" at the COMMAND: prompt.

A menu file can take either of two formats.  The preferred format is a

PDS labelled table file containing file names.  A sample file is shown

below:

NJPL1I00PDS100000000           = SFDU_LABEL

FILE_TYPE                      = TABLE

RECORD_FORMAT                  = STREAM

OBJECT                         = FILE_NAME

TYPE                         = LITERAL

LENGTH                       = 80

ENDOBJECT

NOTE                           = "Sample MENU file for IMDISP"

END

L:\OCEAN\SCBMEAN.IMG

L:\OCEAN\WCMEAN.IMG

L:\PLDS\TMCH01.DAT

L:\PLDS\DEM.DAT

L:\PLANET\SATURN\ATMOS\C3497355.IMG

<END OF FILE>

A second abbreviated form may also be used, where only the filenames

are specified in the ".MNU" file.  In this case the file would only

contain the filenames shown after the END statement in the example

above.

4.5.13 PALETTE

The PALETTE command is used for changing the colors assigned

to  displayable  color  values.  This is  done  by  modifying  an

internal  color  table which assigns intensity  values  for  red,

green  and  blue to a color value, The EGA provides  4  intensity

values  for each of 3 colors, so that 64 different colors can  be

created. However, the EGA pixels are only 4-bits each so only  16

color values can be used simultaneously.

The palette can be modified using the EDIT command and saved

and  loaded from a disk file. The edit mode is invoked by  typing

PALETTE EDIT. With the EGA display, a bar containing 16 colors or

shades  will  be displayed on the bottom of the screen.  A  small

square  will appear in the middle of the center color  box.  This

indicates  that  this  color  value  is  currently  selected  for

editing.  To  select another color value for  editing,  move  the

small square to the appropriate color using the "4" (left  arrow)

and  "6" (right arrow) keys on the numeric keypad. The  color  of

the current color box is changed with the "R", "G", and "B"  keys

(for  red,  green,  and blue). Upper case  letters  increase  the

amount  of  the primary color, and lower case keys  decrease  the

amount of the color. The "8" (up arrow) and "2" (down arrow) keys

increase  and decrease, respectively, the amount of  red,  green,

and  blue simultaneously. Other commands in the palette edit mode

are the  "s" command,  which  will  shift all colors one value to

the right, and the  "x" command, which will exchange color values

(the color setting for color  0 becomes the setting for color 15,

while 15  replaces  0, the  color setting for color 1 becomes the

setting  for 14,  while 14  replaces  1, etc.).  The  up and down

arrow keys can also be used to cycle through the colors available

in  a  given  display  mode  one  at a time.  Type the "." key or

carriage return to exit the edit mode.

PALETTE takes the following keywords and arguments:

EDIT n           to interactively adjust an n-color palette.

See edit description (above) for details

PSEUDOCOLOR n    PS = 0 for default palette,

PS = 1 for gray scale palette,

PS = 2 for pseudo color palette

SAVE filename    saves the specified palette on disk

LOAD filename    loads the specified palette from disk

The  default  palette file for LOAD or  SAVE  operations  is

IMDISP.PAL.  The format of the default color palette when  stored

in a palette file is as shown below:

NJPL1I00PDS000000784            = PDS_SFDU_LABEL

FILE_TYPE                       = TABLE

RECORD_TYPE                     = STREAM

FILE_RECORDS                    =  28

TABLE_ROWS                      =  16

ROW_COLUMNS                     =   4

COLUMN_NAME                     = (COLOR_NUMBER,

RED_VALUE,

GREEN_VALUE,

BLUE_VALUE)

COLUMN_TYPE                     = (INTEGER,INTEGER,INTEGER,INTEGER)

END

0   0   0   0

1  64   0   0

2 128   0   0

3 192   0   0

4 192  64   0

5 192 128   0

6 192 192   0

7 128 192   0

8   0 192   0

9   0 192 128

10   0 128 128

11   0 128 192

12   0   0 192

13 128   0 192

14 192   0 192

15 192 192 192

The  first  column of the palette table is the  color  value

number  for  the display, and columns 2, 3 and  4  represent  the

intensity  of  the  red, green and  blue  (respectively)  primary

colors on a scale of 0 to 255. With the EGA display these  values

are: 0 = off; 64 = low; 128 = medium; 192 = high.

4.5.14 PROFILE

PROFILE  plots the DN values of pixels located along a  line

between  two points in the image, i.e. it creates a graph  of  DN

value versus pixel along the line joining the two points,  Cursor

mode is used to select the two endpoints; typing "." or  carriage

return selects the current cursor position as the endpoint. After

the endpoints are selected a line is drawn between the points and

the graph is plotted at the bottom of the screen, showing the  DN

values  on the x-axis and the positions along the line on the  y-

axis.

4.5.15 QUIT

Exits  the  program. The contents of the image  display  and

refresh buffer are discarded.

4.5.16 REFRESH

REFRESH redisplays the image plane from the refresh  buffer.

When  images are written to the screen they are also  written  to

the  refresh buffer. The REFRESH command writes this buffer  back

to  the screen, thereby erasing any graphics or  text  overlaying

the  image  on the screen. The refresh buffer  contains  as  many

lines  from  the display screen as there is room for  in  memory.

REFRESH  takes,es  no parameters. It can also be used  after  the

FILE prompt mode to redisplay the contents of the refresh buffer.

Refresh is commonly used after the HELP command, or after  a

HISTOGRAM  or PROFILE command to remove text and plots  from  the

display screen.

4.5.17 SAVE

The  save  command  copies  the  REFRESH  buffer  (which  is

normally an exact duplicate of the display screen) to a file.  On

EGA systems the refresh buffer contains only 4 bits per pixel, so

specifying BP = 8 will write the 4-bit pixels in full bytes,  but

does not save full 8-bit pixels even if the input image is an  8-

bit  image,  Use  the COPIM utility program  to  produce  a  full

resolution 16-bit subset of an 8-bit image.

SAVE  takes the following arguments and keywords, where  the

filename must immediately follow the SAVE command:

filename                    filename of image to save

NL n

NS n                        number of lines and samples to save

SL n

SS n                        starting line and sample to save

BP n                        number of bits per pixel in output image

Example: Save a 200 line by 200 sample area at the center of

an  image  in the file SMALL.IMG. First use the FILE  command  to

select and display an input image. Now type:

"SAVE SMALL.IMG SL 100 SS 210 NL 200 NS 200"

to save the portion of the image beginning at line 100 and sample

210 in the new file SMALL.IMG.

4.5.18 SET

SET is used to display and change the DN (pixel value) range

that  is  used  to  compress  the  pixel  values  to  the   range

appropriate  for the display device. The default, which is  reset

for  each  new  file,  is the full range of  DN  values  for  the

particular pixel format (e.g. 0 to 255 for byte).

The results of a SET command will not take effect until  the

next DISPLAY command is performed.

Set takes the following keywords:

DNLOW n

DNHIGH n   where values less than DNlow are set to  color

value 0, values greater than DNHIGH are set to

the  maximum  color value available,  and  the

color  values between are assigned equally  to

the DN values between Dnlow and DNhi.

For  example,  using the default color palette  and  an  EGA

display with 16 color values available, the command "SET DNLO 100

DNHI 164" will assign DN values below 100 to black and DN  values

above 164 to white, and spread the 16 color values between the DN

limits, thus each color value will represent 4 DNs (value 0 =  DN

100 to 103; value 1 - DN 104 to 107, etc.).

If  values other than the default are used then  the  images

will display more slowly because scaling divisions are  required.

SET with no parameters will display the current values.

The  DNLO  and  DNHI values are reset  to  the  minimum  and

maximum for a given pixel size when a FILE command is issued.

4.5.19 STRETCH

STRETCH  sets the palette to a gray scale stretched  between

the two specified pixel values. Stretch is most useful on the PGA

display  where 16 gray levels are available. On the  EGA  display

the  stretch  command  activates the  gray  level  palette  which

provides  only 4 gray levels. Use the PAL PS 0 command to  return

the display to the default palette after performing a stretch.

STRETCH takes the following keywords :

LOW n

HIGh n     the low and high DN values  of the stretch.

These  pixel  values refer to the DN values in  the  current

image display, not necessarily in the original image (scaling may

have  been involved).   For example with the EGA display  the  DN

values in the display range from 0 to 15, even if a byte image is

being displayed.

The STRETCH command is not recommended for use on EGA systems.

4.5.20 SYSTEM

The SYSTEM command is used to send a command to MSDOS.

For  example "SYS L:" will set the L: drive as  the  current

default  drive. If there is not enough memory available  for  the

system command to be executed an error message is displayed.  The

SYS  command will not work on floppy disk systems unless the  file

"COMMAND.COM"  is  present  on the floppy disk  with  the  IMDISP

program.

4.5.21 TEXT

The text command can be used to display a string of text on the

screen.  The text string can be entered on the command line, enclosed

in apostraphies or quotes (e. g., TEXT `This is a text string').

If the text string is not entered on the command line the program will

prompt for the string to be displayed.

Once the command is issued and the text string entered the program is

placed in CURSOR mode so that the display position can be selected.

This is done by using the arrow keys to move the cursor to the desired

location.  After the position is selected, the user types a period (.)

or carriage return and the text will be written on the display screen.

The text command writes over image data in the display and refresh

buffer, thus the underlying portion of the image is lost.

4.5.22 TYPE

The TYPE command is used to type the contents of a text file

on the display screen, using the standard MSDOS type command.

Its argument is passed to MSDOS for execution. At the end of

the type operation the screen will display "Type carriage  return

to  continue:" which will return to the IMDISP command  mode.  If

there is not enough memory available for the system command to be

executed an error message is displayed.

4.6 OTHER UTILITY PROGRAMS

There  are  several utility programs also  provided  on  the

distribution  disk. These programs can read PDS  labeled,  VICAR2

labeled,  or  unlabeled  images. The output  images  are  always

written  with  PDS  ODL  labels.  The  programs  prompt  for  the

necessary input values.

4.6.1 COPIM

COPIM  is a simple image copying program. It' can  copy  the

whole  image, for example to put PDS labels on a foreign  image),

or  extract a window from the image. It can subsample  the  image

with an integral subsampling factor. The program prompts for  the

input  and  output file names, the window to extract  (just  type

return  for  the whole image), and the  subsampling  factor  (the

default  is 1). If the input file is not in VICAR2 or  PDS  image

format  the program will also prompt for the number of lines  and

samples, pixel size and header size of the image.

For example, to extract the first 100 lines and samples from

the file LOGO.IMG the following commands would be used:

COPIM

Input Image: LOGO.IMG

Lines: 350 Samples: 340 Bits per pixel: 1

Output Image: LOGOSUB.IMG

Starting line and sample, number of lines and samples: 1 1 100 100

Line Subsampling factor (1): 1

Sample Subsampling factor(1): 1

4.6.2 CONVERT

CONVERT  is  simple  pixel  format  conversion  program.  It

converts an image to an integer (16 bits), byte (8 bits),  nibble

(4  bits),  or binary (1 bit) image. It can also perform  a  user

specified  scaring. The program prompts for the input and  output

file  names, the output format (must be 1,4,8, or 16  bits),  and

the  input and output numerical ranges for scaling.  The  default

scaling values are appropriate for the data types being used.

For  example, to convert MONTAGE.IMG to an 8-bit  per  pixel

image and scale the output pixels evenly between the values of  0

and 127 the following commands would be used:

CONVERT

Input Image: MONTAGE.IMG

Lines : 350       Samples : 640 Bits per pixel : 4

Output Image: MONTBIG.IMG

Output bits per pixel (1, 4, 8, 16) : 8

Input DN range (O 15) :

Output DN range (O 255) : 0 127

4.6.3 CLEANUP

The  CLEANUP and CLEANUP2 programs are available  separately

with  a  user's guide, executables and required data  files.  The

purpose  of these programs is to clean up "noisy" Voyager  images

by  removing the dark current. The dark current image  represents

the ambient noise in the camera system; therefore, removing  this

noise "cleans up" the original image, making it appear crisper on

the monitor.

Chapter 5

COMMAND SUMMARY

Within this command summary listing, brackets ([])  indicate

optional  parameters or keywords. Filename is the name of a  file

in the current directory or a fully qualified filename which  can

include  drive  letter and pathname. Directory name is  an  MSDOS

directory   path   specification,  a  valid  directory   mask   a

specification  containing wild card characters (*, ?)  per  MSDOS

rules  for wildcard use in the DIRECTORY command. The symbol  "n"

is used to indicate an integer value.

BATCH        [filenamel

CD           [directory name]  [..]

CHDIR        [directory name]  [..]

CURSOR

DIRECTORY    [directory mask]  [/w]

DISPLAY      [SUB n] [SL n] [SS n]  [NL n] [NS n]  [BP n]

[CENT] [DSL n] [DSS n]

ERASE

EXIT

FILE         [filename] ["CD:mm:ss:bb"I

HELP         [command name]

HISTOGRAM    [SUB n] [SL n] [SS n] [NL n] [NS n] [BP n]

LABEL

MENU         [filename]

PALETTE      [EDIT n] [PS n]  [LOAD filename] [SAVE filename]

PROFILE

QUIT

REFRESH

SAVE         filename [SL n] [SS n] [NL n] [NS n] [BP n]

SET          [DNLO n] [DNHI n]

STRETCH      [LO n] [HI n]

SYSTEM       [MSDOS command]

TEXT         "text" or 'text'

TYPE         filename

##########################################################################